Faculty of

Computer Science

CSC230
Computer Networks

Ch_4 Data Link Layer (part 2) _ Ch 5 Network Layer(part 1) _Questions

Assignment #4

Dr. Yasmin Alkady

Dr. Mohammed El-Hossiny

Dr. Mostafa Abdelrahman

Multiple Choice Questions (MCQs):

- Which of the following is NOT a phase in circuit switching?

A) Circuit Establishment
B) Data Transfer
C) Packet Queuing
D) Circuit Disconnect

- What is a characteristic of virtual circuit packet switching?

A) No prior setup is required
B) Packets always arrive out of order
C) All packets follow the same predefined path
D) No headers are needed in packets

- In an example calculation, if the transmission rate is 1 Mbps and a packet size is 8000 bits, what is the transmission time?

A) 1 ms
B) 8 ms
C) 10 ms
D) 0.8 ms

- In circuit switching, what happens during the circuit establishment phase?

A) Packets are transmitted across different paths
B) A dedicated path is set up before data transfer
C) Data is fragmented into multiple packets
D) Nodes process packets dynamically

- What is a key advantage of virtual circuit packet switching over datagram switching?

A) It requires no setup phase before transmission
B) It ensures packets arrive in order
C) It reduces the need for error control
D) It eliminates transmission delays

- If a link has a transmission rate of 320 kbps and a packet size of 8000 bits, what is the transmission time?

A) 0.025 seconds
B) 0.1 seconds
C) 0.5 seconds
D) 1 second

- Which protocols operate at the network layer?

A) TCP and UDP
B) IPv4 and IPv6
C) ARP and MAC
D) HTTP and FTP

- Which field in the IPv4 header is used to classify and prioritize packets?

A) Protocol field
B) Fragment Offset field
C) Differentiated Services Code Point (DSCP) field
D) Header Checksum field

- What is the primary advantage of IPv6 over IPv4?

A) More efficient error correction
B) A larger address space (128-bit addresses)
C) Faster data transmission
D) Elimination of routing tables

- What is de-encapsulation in the network layer?

A) Removing the transport layer header
B) Adding network layer addressing information
C) Extracting the transport layer segment from the network layer packet
D) Encoding bits for transmission

- The 'Header Checksum' field in the IPv4 header is used for:

A) Error detection in the IP header
B) Error correction in the entire packet
C) Assigning priority to packets
D) Identifying the destination address

- Which IPv6 field is equivalent to the TTL field in IPv4?

A) Next Header
B) Hop Limit
C) Payload Length
D) Flow Label

- Which field in the IPv6 header determines the priority of packets?

A) Flow Label
B) Traffic Class
C) Next Header
D) Hop Limit

Problems

- Host A wants to send a 1Mbyte packet to Host B. The propagation speed of bits is Vp=2 *10^8 m/sec. Assume that A and B are connected via a router R. Link AR connects A to R, and link RB connects R to B. Link AR is d1=1Km long and link RB is d2=2Km long.

Suppose that the capacity of each of the C= 2 links is 10 Mbytes/sec and the processing delay in the Router (R) is 10 msec. Find after how long host B will receive the packet. Note the Router (R) must receive the whole packet before being able to forward it.

- In modern packet-switched networks, the source host segments long, application-layer messages (for example, an image or a music file) into smaller packets and sends the packets into the network. The receiver then reassembles the packets back into the original message. We refer to this process as message segmentation. The figure below illustrates a switched network. Consider a message that is 7.5 * 10^6 bits long to be sent from the source to the destination. (Assume header size is negligible relative to the entire message size). Suppose each link is 1.5 Mbps. Focus on transmission delays only and assume all other delay components are negligible.

- Consider sending the message from source to the destination without message segmentation. How long does it take to move the message from the source host to the first packet switch? What is the total time to move the message from source to the destination host?

- Now suppose that the message is segmented into 5000 packets, with each packet being 1500 bits long. How long does it take to move the first packet from source to the first switch? When the first packet is being sent from the first switch to the second switch, the second packet is being sent from the source host to the first switch. At what time will the second packet be fully received at the first switch?

- How long does it take to move the file from source host to destination host when message segmentation is used?

- Compare this result with our answer in part (A) and give at least one advantage and disadvantage of message segmentation.

Important Notice:
All students are required to solve problem P6 on page 99 and problem P31 on page 105 from the E-Book. Click on E-Book Link to open the resource.