Networks’ Documentation

Part 1:

- Addressing Scheme:

VLAN 40 will need 61 IP addresses                                            ->  26 -2=62 hosts /26

VLAN 20 will need 31 IP addresses                                            ->  26 -2=62 hosts /26

VLAN 50  will need 30 IP addresses                                           ->  25 -2=30 hosts /27

VLAN 10 will need 25 IP addresses                                            ->  25 -2=30 hosts /27

VLAN 70 will need 20 IP addresses                                           ->  25 -2=30 hosts /27

VLAN 300 will need 12 IP addresses                                         ->  24 -2=14 hosts /28

VLAN 60 will need 12 IP addresses                                            ->  24 -2=14 hosts /28

VLAN 7 will need 7 IP addresses                                                 ->  24 -2=14 hosts /28

SW-S toMIU-GW will need 7 IP addresses                              ->  24 -2=14 hosts /28

Main-MLS to MIU-MIU-GW will need 2 IP addresses         ->  22 -2=2 hosts /30

From S-MLS to MIU-MIU-GW will need 2 IP addresses    -> 22 -2=2 hosts /30

From S-MLS to MIU-MIU-GW will need 2 IP addresses     -> 22 -2=2 hosts /30

From R-MLS to MIU-MIU-GW will need 2 IP addresses    -> 22 -2=2 hosts /30

Subnet Table:

| Subnet Description | VLAN ID | VLAN Name | Required Number of Hosts | Network Address/CIDR | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Main | VLAN 10 | VLAN10 | 25 | 172.20.5.160/27 | 172.20.5.161/27 | 172.20.5.190/27 | 172.20.5.191/27 |
|  | VLAN 20 | VLAN20 | 31 | 172.20.5.64/26 | 172.20.5.65/26 | 172.20.5.126/26 | 172.20.5.127/26 |
| S | VLAN 300 | VLAN300 | 12 | 172.20.5.224/28 | 172.20.5.225/28 | 172.20.5.238/28 | 172.20.5.239/28 |
|  | VLAN 40 | VLAN400 | 61 | 172.20.5.0/26 | 172.20.5.1/26 | 172.20.5.62/26 | 172.20.5.63/26 |
| N | VLAN 50 | VLAN50 | 30 | 172.20.5.128/27 | 172.20.5.129/27 | 172.20.5.158/27 | 172.20.5.159/27 |
|  | VLAN 60 | VLAN60 | 12 | 172.20.5.240/28 | 172.20.5.241/28 | 172.20.5.254/28 | 172.20.5.255/28 |
| R | VLAN 70 | VLAN70 | 20 | 172.20.5.192/27 | 172.20.5.193/27 | 172.20.5.222/27 | 172.20.5.223/27 |
|  | VLAN 80 | VLAN80 | 7 | 172.20.6.0/28 | 172.20.6.1/28 | 172.20.6.14/28 | 172.20.6.15/28 |
| From Main-MLS to MIU-MIU-GW | - |  | 2 | 172.20.6.32/30 | 172.20.6.33/30 | 172.20.6.34/30 | 172.20.6.35/30 |
| From N-MLS to MIU-MIU-GW | - |  | 2 | 172.20.6.36/30 | 172.20.6.37/30 | 172.20.6.38/30 | 172.20.6.39/30 |
| From S-MLS to MIU-MIU-GW | - |  | 2 | 172.20.6.40/30 | 172.20.6.41/30 | 172.20.6.42/30 | 172.20.6.43/30 |
| From R-MLS to MIU-MIU-GW | - |  | 2 | 172.20.6.44/30 | 172.20.6.45/30 | 172.20.6.46/30 | 172.20.6.47/30 |
| From SW-S to MIU-MIU-GW | - |  | 7 | 172.20.6.16/28 | 172.20.6.17/28 | 172.20.6.30/28 | 172.20.6.31/28 |

Addressing Table:

| Device | Interface | Address | Subnet Mask | Default Gateway | VLAN ID |
| --- | --- | --- | --- | --- | --- |
| Main-MLS | G1/1/1 | 172.20.6.33/30 | 255.255.255.252 |  |  |
|  | VLAN 10 | 172.20.5.161/27 | 255.255.255.224 |  | VLAN 19 |
|  | VLAN 20 | 172.20.5.65/26 | 255.255.255.192 |  | VLAN 20 |
| S-MLS | G1/1/1 | 172.20.6.41/30 | 255.255.255.252 |  |  |
|  | VLAN 300 | 172.20.5.225/28 | 255.255.255.240 |  | VLAN 300 |
|  | VLAN 40 | 172.20.5.1/26 | 255.255.255.192 |  | VLAN 40 |
| N-MLS | G1/1/1 | 172.20.6.37/30 | 255.255.255.252 |  |  |
|  | VLAN 50 | 172.20.5.129/27 | 255.255.255.224 |  | VLAN 50 |
|  | VLAN 60 | 172.20.5.241/28 | 255.255.255.240 |  | VLAN 60 |
| R-MLS | G1/0/5 | 172.20.6.45/30 | 255.255.255.252 |  |  |
|  | VLAN 70 | 172.20.5.193/27 | 255.255.255.224 |  | VLAN 70 |
|  | VLAN 80 | 172.20.6.1/28 | 255.255.255.240 |  | VLAN 80 |
| MIU-MIU-GW | Gig0/0 | 209.165.200.226 | 255.255.255.240 |  |  |
|  | Gig0/1 | 64.100.1.2/27 | 255.255.255.224 |  |  |
|  | Gig0/0/0 | 172.20.6.30/28 | 255.255.255.240 |  |  |
|  | Gig0/1/0 | 172.20.6.34/30 | 255.255.255.252 |  |  |
|  | Gig0/2/0 | 172.20.6.42/30 | 255.255.255.252 |  |  |
|  | Gig0/3/0 | 172.20.6.46/30 | 255.255.255.252 |  |  |
| ISP | Gig 0/0 | 209.165.200.225 | 255.255.255.240 |  |  |
|  | Gig 0/1 | 64.100.1.1 | 255.255.255.224 |  |  |
|  | Gig 0/2 | 64.100.2.1 | 255.255.255.224 |  |  |
| Branch-GW | Gig 0/0.2 | 192.168.2.1 | 255.255.255.0 |  |  |
|  | Gig 0/0.3 | 192.168.3.1 | 255.255.255.0 |  |  |
|  | Gig 0/1 | 64.100.1.2 | 255.255.255.224 |  |  |
| Wireless Home Router | G0/1 | 64.100.2.2 | 255.255.255.224 |  |  |
|  | Router IP | 192.168.10.1 | 255.255.255.128 |  |  |

Hosts Address:

| PC-number | VLAN ID | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| MIU |  |  |  |  |
| PC-1 |  | 172.20.5.190/27 | From DHCP Server | 172.20.5.161/27 |
| PC-2 |  | 172.20.5.162/27 | From DHCP Server | 172.20.5.161/27 |
| PC-3 |  | 172.20.5.126/26 | From DHCP Server | 172.20.5.65/26 |
| PC-4 |  | 172.20.5.238/28 | From DHCP Server | 172.20.5.225/28 |
| PC-5 |  | 172.20.5.226/28 | From DHCP Server | 172.20.5.225/28 |
| PC-6 |  | 172.20.5.62/26 | From DHCP Server | 172.20.5.1/26 |
| PC-7 |  | 172.20.5.158/27 | 255.255.255.224 | 172.20.5.129/27 |
| PC-8 |  | 172.20.5.254/28 | 255.255.255.240 | 172.20.5.241/28 |
| PC-9 |  | 172.20.5.222/27 | 255.255.255.224 | 172.20.5.193/27 |
| PC-10 |  | 172.20.6.14/28 | 255.255.255.240 | 172.20.6.1/28 |
| MIU_ Branch 1 |  |  |  |  |
| PC-11 |  | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-12 |  | From DHCP Server | From DHCP Server | From DHCP Server |
| Wireless Home Network |  |  |  |  |
| Laptop |  | 64.100.2.3/27 | 255.255.255.224 | 64.100.2.2 |
| Tablet |  | 64.100.2.4/27 | 255.255.255.224 | 64.100.2.2 |
| Smartphone |  | 64.100.2.5/27 | 255.255.255.224 | 64.100.2.2 |
| Servers |  |  |  |  |
| DHCP Server |  | 172.20.6.18/28 | 255.255.255.240 | 172.20.6.30/28 |
| Email Server |  | 172.20.6.19/28 | 255.255.255.240 | 172.20.6.30/28 |
| Web server |  | 172.20.6.20/28 | 255.255.255.240 | 172.20.6.30/28 |
| DNS server |  | 172.20.6.21/28 | 255.255.255.240 | 172.20.6.30/28 |
| NTP and Syslog server |  | 172.20.6.22/28 | 255.255.255.240 | 172.20.6.30/28 |

Final Implementation Documentation for Network Project

Part 2: Build the Network and Configure Basic Device Settings and Interface Addressing (10%)

What we did:

- Assigned full IPv4 addresses to all PCs manually based on the Addressing Table.

- Configured hostnames, passwords, console security, MOTD banners, and SSH for all routers and switches.

- Ensured domain lookup was disabled to prevent accidental DNS lookups.

Commands used:

hostname [NAME]                          // Set device hostname

no ip domain-lookup                     // Prevent incorrect DNS lookups

enable secret MIU1234567                // Set enable password

line console 0

password CSC1234567                   // Console access password

login

banner motd #Unauthorized access#       // MOTD banner

ip domain-name miu.edu.eg               // Define domain for SSH

crypto key generate rsa                 // Generate RSA key for SSH

ios dhcp excluded-address ...           // Reserve Ips

Problems encountered:

- None, this step was completed smoothly.

Part 3: Network Infrastructure Settings (10%)

What we did:

- Created VLANs across switches.

- Verified VLANs with show vlan brief.

- Created access mode links between switches and pcs

- Created trunk links using switchport trunk , switchport mode trunk.

- Used interface range and channel-group commands for EtherChannel setup.

- Set up Inter-VLAN routing:

  - L3 switches: Configured routed ports and SVI interfaces.

  - Branch-GW: Used subinterfaces on GigabitEthernet0/0 with encapsulation dot1Q for VLAN2 and VLAN3.

Commands used:

vlan [x]                          // Create VLANs

interface range f0/1 – [x]

switchport access vlan [x]            // Assign ports to VLAN

interface range g0/1 – [x]

channel-group 1 mode active           // EtherChannel

interface port-channel 1

switchport mode trunk                 // Trunk

interface g[x]/[x]

no switchport

ip address 172.20.x.x ...             // Routed interface

Problems encountered:

- EtherChannel initially didn’t work due to missing mode active/passive.

Part 4: Configure Routing Protocols (10%)

What we did:

- OSPF (Process ID 100) configured on MIU-GW, Main-MLS, S-MLS.

  - Included  default route propagation.

- EIGRP (AS 10) configured on MIU-GW, N-MLS, R-MLS.

  - Default route and directly connected network advertisement.

- Route Redistribution between OSPF and EIGRP on MIU-GW.

- Configured static routes on MIU-GW and Branch-GW pointing to ISP.

Commands used:

router ospf 100     //start ospf proccess

router-id 1.1.1.1       //giving router its unique id

network 172.20.0.0 0.0.63.255 area 0        //puts the network in ospf area 0

no passive-interface g0/0                                                 interfaces except G0/0

default-information originates                                  // Propagates the default route through

OSPF to other routers.

router eigrp 10                                                      // Enables the EIGRP process using

Autonomous System (AS) number 10.

network 172.20.0.0                                         //puts networks in branch to eigrp

no passive-interface g0/1                             interfaces that don’t need it.

redistribute ospf 100 metric 10000 1 255 1 1500    //tells EIGRP to accept routes learned

from OSPF.

redistribute eigrp 10 subnets into ospf 100                // Tells OSPF to accept routes

learned from EIGRP.

ip route 0.0.0.0 0.0.0.0 64.100.1.1                                    // Static default route

Problems encountered:

- Default routes weren’t advertised initially. Redistribution commands fixed it.

Part 5: Configure DHCP Server (5%)

What we did:

- Configured DHCP pools on Main Branch and Branch-GW.

- Used ip dhcp excluded-address to reserve static IPs.

- Branch-GW used DHCP pools B-LAN2 and B-LAN3.

Commands used( doing the same in both MIU-GW and BRANCH-GW):

ip dhcp excluded-address [172.20.5.0 ]  [0.0.3.255]x3   // Prevents DHCP fromassigning

critical addresses like gateway/router/SVI.

ip dhcp pool VLAN[x]                                                 // Creates a DHCP pool for VLAN[x]

network 192.168.2.0 255.255.255.0

default-router 192.168.2.1                                // gateway IP

dns-server 172.20.5.10                                    // internal DNS

domain-name miu.edu.eg                              // sets internal domain suffix

Problems encountered:

- Incorrect mask on one pool. Fixed by rechecking the config.

Part 6: NAT Configuration (5%)

What we did:

- Configured Dynamic NAT with PAT and Static NAT for the web server.

Commands used:

- access-list 1 permit 172.20.0.0 0.0.63.255// Allows inside LAN addresses

- ip nat inside source list 1 interface g0/0 overload

// Enables PAT (Port Address Translation) using the G0/0 interface’s public IP. All internal users share one public IP with different port numbers.

- ip nat inside source static 172.20.5.10 209.165.200.227

// Maps the internal IP of the Web Server (or DNS, Email, etc.) to the public IP so it can be accessed from the outside.

- interface g0/0    // The interface connected to the ISP

- ip nat outside

- interface g0/1   // The interface facing the internal network (LAN).

- ip nat inside

Problems encountered:

- NAT didn’t work until ACL 1 was corrected.

Part 7: Network Management Configuration (10%)

What we did:

- Set up NTP, Syslog, DNS, Email, and Web Servers.

Commands used:

ntp server 172.20.5.8   // Synchronizes time with an internal NTP server

logging 172.20.5.7 // Sends log messages to the Syslog Server.

ip name-server 172.20.5.6   // Maps names (like www.miu.edu.eg) to IPs.

Problems encountered:

- DNS initially not resolving until zone file was updated.

Part 8: IPsec VPN (10%)

What we did:

- Site-to-site IPsec VPN between MIU-GW and Branch-GW.

Commands used (Branch-GW and MIU-GW):

access-list 130 permit ip 192.168.2.0 0.0.0.255 172.20.5.0 0.0.63.255

// Tells the router which traffic to encrypt (from MIU to Branch).

- crypto isakmp policy 10                        //Defines IKE Phase 1 policy

- encryption aes 256                                  // AES encryption

- authentication pre-share                   // pre-shared key

- group 5                                                        // Diffie-Hellman group 5

- crypto isakmp key cisco45 address 209.165.200.225     //shares passkey

- crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

// Defines IPsec transform set (encryption and authentication for the VPN tunnel)

- crypto map VPN-MAP 10 ipsec-isakmp

// Defines the full VPN config and binds it to access-list 130,

- set peer 209.165.200.225             // Defines the full VPN config and binds it to

access-list 130, the peer, and transform-set

- set transform-set VPN-SET

- match address 130

- interface g0/1                                // Activates the VPN on the outbound interface.

- crypto map VPN-MAP

Problems encountered:

- ACL direction issues and wildcard mismatch blocked traffic.

- Crypto map didn’t apply due to misnaming.

- It works just one way not both ways

Part 9: Wireless Configuration (10%)

What we did:

- Configured the SSID and WPA2 key on Wireless Router.

SSID: MIU-CSC230 Passphrase: miu_csc230

Problems encountered:

- Device didn’t connect due to SSID case mismatch.

Part 10: Verify Connectivity (10%)

What we did:

- Verified with ping, traceroute, show arp ,show ip route, show crypto isakmp sa, show crypto ipsec sa

Commands used:

Show run

ping [IP]

traceroute [IP]

show ip route

show arp

show ip interface brief

show crypto isakmp sa

show crypto ipsec sa

Problems encountered:

- VPN didn't initiate until real traffic (e.g. ping) passed.

Part 11: Documentation and Handover (10%)

What we did:

- Connectivity check:

- Routing table confirmed.

Commands used:

show run

show ip route

show crypto isakmp sa

show crypto ipsec sa

Final Note: All functionalities work after debugging VPN ACLs and crypto map mismatches. The entire topology is fully functional and secure.